# SCBA/RIT Instructor Notes

# SCBA/PPE donning

- Emphasize doing it correctly before quickly.

- Emphasize seated SCBA donning over other methods.

- Emphasize the need for buckling the waist belt, SCBA can be used for dragging a downed firefighter.

- Practice putting the mask on while using structural firefighting gloves.

- Steps for donning seated are included in this document to be used as a handout, if desired, or refer to full ‘Donning SCBA’ word document, beginning on page 3, for donning steps.

- Standard at end of the training is for the firefighter to be able to don the SCBA equipment and be ready to be ‘on air’ in 1 minute or less, while wearing full turnout gear.

# Rit CPR

- Watch video prior to class, so you are familiar with the steps outlined in the power point.

- The steps in the video go by pretty fast, it may be beneficial to watch it more than once.

- Emphasize the importance of Rescuer 1’s job of securely holding the straps and coat when setting up for the pull down.

- Try to have at least one group demonstrate live. If time allows, have more do the drill, ideally as Rescuer 1 and 3 or 4.

# Seated SCBA Donning

  - Procedure to don seat mounted SCBA

    - Open the cylinder valve fully

    - Check the cylinder and the remote gauge readings

      - Pressure readings must be within 100 psi

      - Needles on both gauges indicate the same pressure for gauges without 100 psi increments

    - Position your body in the seat with your back firmly against the SCBA and release the SCBA hold-down device

    - Insert your arms through the shoulder straps

    - Fasten the chest straps, buckle the waist strap, and adjust the shoulder straps

    - Fasten your seat belt before the apparatus moves

    - Don the facepiece and check the facepiece seal

    - Connect the air supply to the facepiece when you are ready to use it

    - Activate the PASS device manually if it is not integrated in the system (See Township Fire SOG 2-1-7 for specific alerts for this PASS device)

    - Don the hood, the helmet, and the gloves

  - Exit the fire apparatus carefully to avoid slips or falls due to the extra weight on your back